Steps to Access ‘App’ Interface

- Download ‘Bluefy’ on iOS App Store

- Enter hhtps://www.theonlywgt.com into the search bar.

- Ensure you reach the Connect to MCU page

- Power ESP32 and flash Arduino code. The current code to do this is saved as blu_test.ino

- Click ‘Connect’ on the app and select the ESP32. Wait for the connection indicator to turn green.

  - Tip: Send a test message using the Payload text line. Send using RX

- Click the blue Open button to enter the app.

- Proceed to send data to the MCU using Packets

  - Basic waveforms send almost instantly

  - Arbitrary waveforms take ~2.5 minutes to send

Notes for future Implementation:

- Improve speed ?

- Add download CSV function and check sent data makes the intended waveforms

Bugs Found:

Standard Waveforms

- Change Amplitude to VPP

- Change Amplitude range to 0-10

- Offset -5V to 5V

- Frequency change to Hz/KHz dropdown

- Add a phase slider from -180-180 degrees for standard waveforms.

- Reduce Y-Axis Scale to be -5 to 5

- Increase X-axis scale to be 0 to 100

- Remove Start Button

- Make graph larger on iphone

Arbitrary Waveform

- Fix Phase from -180 to 180 on Arb Editor

- 0 to positive time restrictions (throw an error, not a valid waveform (?))

- No negative values  (1st and 4th Quadrant Only on drawable tab)

- Limit y scale to +=5 Volts

- Fix Drawable Tab Discontinuities

  - How to connect multiple waveforms when they end in different VPP

- Replace Drawable Tab to "Arbitrary" so there are only two tabs

- Limit Freq to 100kHz and Positive numbers

- Limit VPP to +5 and -5V

Overall Need to-Dos

- Fix CSV to download from phone to USB

- Change AWG to Func Generator

- Externally download points and rebuild to verify the correct waveform is being sent

Arduino Code

- Change Arduino Code to save CSV of points to check data sent

- Multiply data by Vpp and save to CSV easier